Voltage measurement and Displaying in LCD

Learning Objective: -

- Measuring voltage using 8051 with interfacing ADC 0808

- Displaying the voltage value in LCD display

- Indicating the range of voltage using LEDs

Inputs and Outputs: -

- Input is Potentiometer

- Outputs are LEDs and LCD display

Logic: -

- Connect ADC, LCD, LEDs to 8051 Microcontroller

- The POT and 5v source are connected to ADC 0808

- The ADC will convert the voltage (analog signal) to Digital signal

- That signal is fed to 8051 and processes the signal and send the value to the LCD

- The LCD will display the Voltage applied to ADC and will show the no of LEDs are glown

  - If the voltage is more than 3v, two LEDs will glow

  - If voltage is in between 2v -3v, then 1 LED will glow

  - If voltage is less than 2v, no LEDs will glow

- The maximum voltage that can be measured is 5v DC

Common Mistakes: -

- Improper connection from ADC to 8051

- Not Assigning all port that have been used

- Not displaying decimal point in LCD while displaying voltage

- Checking Active high or low port and assigning the values later

Result: -